SECTION 1: IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND THE COMPANY/UNDERTAKING

1

.

1

.

Product identifier

1

.

2

.

Relevant identified uses of the substance or mixture and uses advised against

Putties (component A) based on non-saturated polyester  resins used for levelling scratches and pits before spraying. For

professional use in car refinish.

1

.

3

.

Data of the supplier Safety Data Sheet

RADEX-EUROPE Ltd..

Tel: +371 67387778

Fax:+371 67387789

Riga, LV-1005, LATVIA

www.radex-auro.com

Person responsible for the Safety Data Sheet

info@radex-europe.lv

1

.

4

.

Emergency telephone number

SECTION 2: HAZARD IDENTIFICATION

2

.

1

.

Classification of the substance or mixture

Classification 1999/45/EC:

The mixture was classified as dangerous pursuant to current regulations - see section 15. Harmful mixture. Harmful by

inhalation. Irritating to eyes and skin. Flammable liquid, avoid contact with strong oxidants.

2

.

2

.

Label elements:

Contains:

styrene

Signs:

Risk symbols:

Xn

Harmful

Risk index:

R10

Flammable

R20

Harmful by inhalation.

R36/38

Irritating to eyes and skin.

Safety index:

S(2-)

Keep out of the reach of children.

S22

Do not breathe dust.

S23

Do not breathe vapour.

S36

Wear suitable protective clothing.

S46

If swallowed, seek medical advice immediately and show this container

or label

S51

Use only in well-ventilated areas.

2

.

3

.

Other hazards

Styrene fumes form explosive mixtures with air. Fumes are heavier than air and accumulate close to the ground level and in

lower parts of rooms.

Styrene polymerization may occur under  the influence of high temperature or  as a result of contact with strongly oxidizing

agents, peroxides, strong acids, bases, metal salts, copper and its alloys. Styrene polymerization is a highly exothermic process.

MATERIAL SAFETY DATA SHEET

Date of issue: 18.09.2012. No. 09-2012	Page: 1 of 7

RADEX PUTTIES: UNI, SOFT, EASY LIGHT, EASY LIGHT PLUS,  GREEN GLASS, ALU, PALSTIC FLEX, SPRAY FILLER

RADEX PUTTIES: UNI, SOFT, EASY LIGHT, GREEN GLASS, ALU, PALSTIC FLEX, SPRAY FILLER

Uriekstes street 3

+371 67387778 (from 09.00 to 18.00)

SAFETY DATA SHEET

Date of issue: 18.09.2012.

Number: 09-2012

Page: 2 of 7

SECTION 3: COMPOSITION/INFORMATION ON INGREDIENTS

3

.

1

.

Substances

Not applicable.

3

.

2

.

Mixtures

Product identification

UNIVERSAL

PUTTY,

UNI;

SOFT

PUTTY,

UNISOFT;

MULTIFUNCTIONAL

PUTTY,

SOFT

PLUS;

FILLING

PUTTY,

ECONOMY;

PUTTY

FOR

TIN  AND

ALUMINIUM,

PROFICYNK;

FINISHING  PUTTY,

FINISH;

INDUSTRIAL

PUTTY,

TECH

PLUS;

PUTTY WITH ALUMINIUM POWDER, ALU; SOFT LEVELLING PUTTY,

UNI PLUS

Substance name

Identification numbers

Classification and marking

Concentrati

on

[wt%]

Styrene

EC: 202-851-5

CAS: 100-42-5

Index no.: 601-026-00-0

Registration no.:--

Classification 67/548/EEC:

R10

Xn; R20

Xi; R36/38

1

2

.

5

-

1

4

Classification 1272/2008/EC:

Flam. Liq. 3; H226

Acute Tox. 4; H332

Eye Irrit. 2; H319

Skin Irrit. 2; H315

Marking:

GHS02; GHS07; Wng; H226, H332, H319,

H315

Full text of the phrases identifying the types of hazards and R phrases is provided in section 16.

SECTION 4: FIRST AID MEASURES

4

.

1

.

Description of first aid measures

General information:

See section 11 of the Safety Data Sheet.

Inhalation:

Take the victim outside into fresh air, ensure quiet surrounding, in case of no breath, apply artificial respiration. Call a doctor.

Skin:

Take off contaminated clothing. Rinse contaminated skin with plenty of lukewarm  water for about 15 minutes. If  irritation

persists, consult a doctor.

Eyes:

Rinse immediately with plenty of lukewarm water for about 15 minutes, avoid strong water jet-risk of cornea damage, consult a

doctor.

Alimentary tract:

Do not provoke vomiting (choking risk). Rinse mouth with water. If conscious, administer 1-2 glasses of warm water. Call a

doctor.

Person giving first aid should wear medical gloves.

4

.

2

.

Most important symptoms and effects, both acute and delayed

Styrene foams in low concentrations may cause eye lacrimation, metallic taste in mouth, painful and reddened conjuctivas, in

higher concentrations - cough, dizziness, disequilibrium.

4

.

3

.

Indications of any immediate medical attention and special treatment needed

Special measures allowing for specialist and immediate aid should be available in the place of work.

SAFETY DATA SHEET

Date of issue: 19.09.2012.

Number: 09-2012

Page: 3 of 7

SECTION 5: FIREFIGHTING MEASURES

5

.

1

.

Extinguishing media

Powder, foam resistant to alcohols, carbon dioxide, water mist.

5

.

2

.

Special hazards arising from the substance or mixture

Styrene polymerization may occur under  the influence of high temperature or as a result of contact with strongly oxidizing

agents, peroxides, strong acids, bases, metal salts, copper and its alloys. Styrene polymerization is a highly exothermic process.

Fire may cause generation of carbon dioxide and other toxic gases.

5

.

3

.

Advice for firefighters

Fire-fighting teams should wear self-contained breathing apparatus and light protective clothing. Cool  adjacent tanks by

spraying water at a safe distance.

SECTION 6: ACCIDENTAL RELEASE MEASURES

6

.

1

.

Personal precautions, protective equipment and emergency procedures

For persons not being the members of aid giving staff:

Eliminate sources of ignition. Ensure sufficient ventilation of the room. Avoid direct contact with the released substance. Avoid

contact with skin and eyes. Personal protection measures - section 8 of the Safety Data Sheet.

For persons giving aid:

Persons giving aid should wear  protective clothing made of coated, impregnated fabric, protective gloves (viton), tight

protective glasses and breathing apparatus: gas mask with A type absorber.

6

.

2

.

Environmental precautions

Prevent leakage to the sewage system, surface waters, underground waters and soil.

6

.

3

.

Methods and materials for containment and cleaning up

Stop the leakage (close the liquid inflow, seal), place damaged containers in an emergency container, remove the liquid

mechanically and place it in an emergency container. In case of large leakage, embank the area. In case of small amounts,

collect with the use of a binding agent (e.g. mica, diatomaceous earth, sand).

6

.

4

.

Reference to other sections

Personal protection measures - see section 8 of the Safety Data Sheet.

Disposal considerations - see section 13 of the Safety Data Sheet.

SECTION 7: HANDLING AND STORAGE

7

.

1

.

Precautions for safe handling

Keep away from heat and fire sources. Prevent leakage to the sewage system, surface waters, underground waters and soil.

Use in well ventilated rooms. Do not smoke. Do not inhale fumes. Avoid contact with skin and eyes. Take precaution measures

against electrostatic discharge. Use personal protection measures - section 8 of the Safety Data Sheet.

7

.

2

.

Conditions for safe storage, including any incompatibilities

Store in tightly sealed, original containers. Do not store near large amounts of organic peroxides and other strong oxidants.

Take precaution measures against electrostatic discharge. Store in cool, well ventilated rooms. Protect from low temperatures,

influence of sunrays and heat sources.

7

.

3

.

Specific end use(s)

Putties (component A) based on non-saturated polyester  resins used for  levelling scratches and pits before spraying. For

professional use in car refinish, taking into consideration the information included in sections 7.1 and 7.2.

SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION

8

.

1

.

Control parameters

Styrene CAS 100-42-5 according to:



TRGS 900:

MAK: 20ppm, MAK: 86 mg/m , 2(II),DFG, Y

3



Adopted

National  Exposure

Standards

for

Atmospheric

Contaminants

in

the

Occupational  Environment

[NOHSC:1003(1995)]:

TWA 100 mg/m 430 mg/m , STEL 250ppm, 1080 mg/m

3

,

3

3

SAFETY DATA SHEET

Date of issue: 19.09.2012.

Number: 09-2012

Page: 4 of 7

SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION

8

.

2

.

Exposure control

Respiratory tract protection:

Gas mask with A type absorber (EN 141).

Hand protection:

Protective gloves PN-EN 374-3 (viton, 0.7 mm thick, penetration time > 480 min, nitrile rubber, o.4 mm thick, penetration time >

3

0

min)

Eye protection:

Tight protective glasses.

Skin protection:

Proper protective clothing (coated, impregnated fabrics).

Workplace:

Fixed fume extraction and general ventilation.

Persons suffering from respiratory tract hypersensitivity (e.g. asthma, chronic respiratory tract inflammation)

should avoid contact with the product.

Environmental exposure control:

Prevent leakage to the sewage system, surface waters, underground waters and soil.

Section 9: PHYSICAL AND CHEMICAL PROPERTIES

9

.

1

.

Information on basic physical and chemical properties

Physical state

highly viscous liquid

Colour

beige, light grey, white, dark grey with visible aluminium plates

Odour

slightly sweet to strong

Odour threshold

0

.

4

3

mg/m3 (styrene)

pH

not applicable

Melting/freezing point

-3

0

°C

Boiling point

1

4

6

°C

Flash point

3

0

°C

Autoignition point

4

9

0

°C

Breakdown point

not specified

Evaporation rate

not specified

Flammability (solid, gas)

not applicable

Explosion limits

% bottom 1.1 vol%  top: 8.0 vol% (styrene)

Vapour pressure

about 7.3 hPa (20°C) (styrene)

Vapour density (with regard to air)

3

.

6

(styrene)

Density

1

.

8

5

-

1

.9

5

g/cm

3

(20°C)

Solubility (in water)

Very poor

N-o

ctanol/water division ratio

3

.

2

(styrene)

Viscosity (rotation rheometer)

2

0

0

0

0

– 50000 mPas

Explosive properties

not applicable

Oxidizing properties

not applicable

SECTION 10: STABILITY AND REACTIVITY

1

0

.

1

.

Reactivity

The product is not reactive under normal conditions.

1

0

.

2

.

Chemical stability

The product remains stable under normal conditions.

1

0

.

3

.

Possibility of hazardous reactions

Styrene polymerization may occur under  the influence of high temperature or  as a result of contact with strongly oxidizing

agents, peroxides, strong acids, bases, metal salts, copper and its alloys. Uncontrolled polymerization in a closed container

might result in an explosion. Carbon monoxide and other toxic gases are generated as a result of thermal decomposition.

SAFETY DATA SHEET

Date of issue: 19.09.2012.

Number: 09-2012

Page: 5 of 7

SECTION 10: STABILITY AND REACTIVITY

1

0

.

4

.

Conditions to avoid

Flammable product. Avoid contact with strongly oxidizing agents, peroxides, strong acids and bases. Avoid generation and

accumulation of static electricity. Protect from the influence of sunrays and heat sources.

1

0

.

5

.

Incompatible materials

Avoid contact with large amounts of organic peroxides, strong acids and bases as well as other strong oxidants.

1

0

.

6

.

Hazardous decomposition products

Carbon monoxide and other toxic gases are generated as a result of thermal decomposition.

SECTION 11: TOXICOLOGICAL INFORMATION

1

1

.

1

.

Information on toxicological effects

No experimental data available on the preparation. Evaluation was performed based on the data on dangerous ingredients

included in the preparation.

a) Acute toxicity

Styren

LD50 (rat, oral) – 5000 mg/kg

LC50 (rat, inhalation) – 24000 mg/mі (4 h)

TCL0 (human, inhalation) – 2600 mg/mі

LCL0 (human, inhalation – 43000 mg/mі

b)

Irritating effect

Skin: irritating to skin and mucous membrane

Eyes: irritating effect

c) Caustic effect

The mixture has not been classified as caustic. No available data confirming the hazard class.

d)

Allergenic effects

The mixture has not been classified as allergenic. No available data confirming the hazard class.

e) Toxicity for repeated exposure

Styrene foams in low concentrations may cause eye lacrimation, metallic taste in mouth; in concentrations of ca. 800 mg/mі -

painful and reddened conjuctivas, in higher concentrations - cough, dizziness, disequilibrium. Prolonged exposure causes

drowsiness, disturbances of consciousness, possible paralysis of the respiratory centre.

f) Cancerogenity

The mixture has not been classified as cancerogenic. No available data confirming the hazard class.

g)

Mutagenity

The mixture has not been classified as mutagenic. No available data confirming the hazard class.

h)

Harmful effect on reproduction

The mixture has not been classified as having any harmful effect on reproduction. No available data confirming

the hazard class.

Exposure methods:

Inhalation: Harmful in case of inhalation.

Skin: Irritating effect.

Eyes: irritating effect.

If swallowed, the substance may cause irritation of the alimentary tract, nausea, vomiting and diarrhoea.

Poisoning symptoms:

Headache and vertigo, fatigue, decreased muscle power, drowsiness and, in exceptional instances, loss of consciousness. If

swallowed, the substance may cause irritation of the alimentary tract, nausea, vomiting and diarrhoea. It has depressing effect

on the central nervous system.

SECTION 12: ECOLOGICAL INFORMATION

No experimental data available on the preparation. Evaluation was performed based on the data on dangerous ingredients

included in the preparation.

SAFETY DATA SHEET

Date of issue: 19.09.2012.

Number: 09-2012

Page: 6 of 7

SECTION 12: ECOLOGICAL INFORMATION

1

2

.

1

.

Toxicity

styrene

Acute toxicity for fish:

LC50 4-10 mg/l/96h

Acute toxicity for crustacea: Daphnia manga EC50/24 182 mg/l/24h

Number in the catalogue of water hazardous substances:

1

8

7

Water hazard class:

2

1

2

.

2

.

Persistence and degradability

styrene

Biodegradability: 80% (closed bottle test)

1

2

.

3

.

Bioaccumulative potential

Log Pow: 2.96 (OECD 107)  - low bioaccumulation ability

styrene

1

2

.

4

.

Mobility in soil

Product very poorly soluble in water.

1

2

.

5

.

Results of PBT and vPvB assessment

No available data.

1

2

.

6

.

Other adverse effects

No available data.

SECTION 13: DISPOSAL CONSIDERATIONS

1

3

.

1

.

Waste treatment methods

The product must be disposed of in compliance with proper local and statutory regulations with regard to waste - see point 15.

The product should be disposed with entities which are authorised to conduct activity in the area of collecting, recycling or

utilization of waste.

Product remains:

Unhardened remains of the product are harmful waste. Do not dispose the product into the sewage system. Do not store with

communal waste. Remove the remains of the mixture carefully and harden with the use of the proper B component, a (waste)

hardener from the set. The hardened product is not harmful waste.

CAUTION: harden the remains in small portions and keep them away from flammable products. High amounts of heat are

released during chemical reaction!

Contaminated container:

A container containing unhardened remains of the product is harmful waste. Do not store with communal waste.

The contaminated container should be disposed with entities which are authorized to collection, recover or disposal.

SECTION 14: TRANSPORT INFORMATION

ADR/RID

IMO/IMGD

IATA-DGR

1

4

.

1

.

UN number

1

8

6

6

1

8

6

6

1

8

6

6

1

4

.

2

.

UN proper shipping name

RESIN SOLUTION, flammable

1

4

.

3

.

Transport hazard class(es)

3

3

3

1

4

.

4

.

Packaging group

III

III

III

1

4

.

5

.

Environmental hazards

none

none

none

1

4

.

6

.

Special precautions for user

Do not transport with materials of class 1 (excluding materials of class 1.4S) and some materials of classes 4.1 and

5

.

2

.

During transport, avoid direct contact with materials of classes 5.1 and 5.2. Do not use an open flame and do not

smoke.

1

4

.

7

.

Transport in bulk according to Annex II of MARPOL 73/78 Convention and the IBC Code

Not applicable.

SAFETY DATA SHEET

Date of issue: 19.09.2012.

Number: 09-2012

Page: 7 of 7

SECTION 15: REGULATORY INFORMATION

1

5

.

1

.

Safety, health and environmental regulations/legislation specific for the substance or mixture

Directive 67/548/EWG(2006/121/WE)

Directive 91/155/EWG (2001/58/WE)

Directive 1999/45/EC (2006/8/WE)

REACH - Regulation 2006/1907/WE

CLP - Regulation 1272/2008/WE

1

5

.

2

.

Chemical safety assessment

Not performed

SECTION 16: OTHER INFORMATION

Full text of the phrases identifying the types of hazards and R phrases mentioned in sections 2-15

R10 Flammable

R20 Harmful by inhalation.

R36/38 Irritating to eyes and skin.

Flam.Liq.3 Liquid, flammable substances, category 3

H226 Flammable liquid and vapours

Acute Tox. 4. Acute toxicity, category 4

H332 Harmful effect as a result of inhalation

Eye Irrit. 2. Irritating effect on eyes, category 2

H319 Eye irritating effect

Skin Irrit. 2. Irritating effect on skin, category 2

H315 Skin: irritating effect (category 2).

Explanation of the abbreviations and acronyms used in the Safety Data Sheet

GHS02 – "flame" pictogram code

GHS07 – "exclamation mark" pictogram code

Wng – "caution" warning notice codes

CAS  no – numerical  symbol  ascribed to a chemical substance by the American organization, Chemical Abstracts Service

(CAS).

EC no. – a number ascribed to a chemical substance in the European List of Notified Chemical Substances (ELINCS) or a

number in the European Inventory of Existing Chemical Substances mention in "No-longer polymers" publication (EINECS)

MPC – maximum permissible concentration of health hazardous substances in the work place

MPIC – maximum permissible instantaneous concentration

MPCC - maximum permissible ceiling concentration

PCB - permissible concentration in biological material

UN number - four-digit identification number of a substance, preparation or product pursuant to UN model regulations

ADR – European agreement on international road transport of hazardous materials

IMO – International Marine Organization

RID – Regulations for international rail transport of hazardous materials

IMDG-Code – International marine code for hazardous materials

ICAO /IATA – Technical Instructions for Safe Air Transport of Hazardous Materials

The information is based on our current knowledge. This document shall not constitute warranty for product characteristics.

Classification of the mixture results from the application of the classification rules contained in Directive 1999/45/EC.

Other sources of information

ESIS European Chemical Substances Information System

TOXNET Toxicology Data Network

IUCLID International Uniform Chemical Information Database

Changes: General update

Trainings:

With regard to handling, health and safety while working with hazardous substances and mixtures.

With regard to transport of hazardous goods pursuant to the requirements of ADR regulations.

Issued by: RADEX-EUROPE Ltd.